W/DP RD-Q4187-01-A0

Customer Name:

Standard:      RM-7202-868

P/N:

| Plot |  |  | The Label |
| --- | --- | --- | --- |
| Drew | Audited | Approved | Stamp, please! Thanks! |
| Date： |  |  |  |

### Guangdong Dapu Telecom Technology Co.,Ltd

Bldg16,.N.Ind.Zone,SSL Industry Park, Dongguan City, Guangdong Province, China

TEL：0086-0769-88010888  FAX：0086-0769-81800098

The Parameter of Product

1. Running Parameters

| MODEL ： RM-7202-868 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Description |  | Parameters |  |  | Unit | Test Condition |
|  |  | Min. | Typ. | Max. |  |  |
| Supply voltage |  | 1.8 | 3.3 | 3.7 | V |  |
| Operational temperature range |  | -40 |  | 85 | ℃ |  |
| Power consumption | Sleep Idle |  | 2 |  | uA |  |
|  | Standby |  | 2 |  | mA |  |
|  | Receive Mode |  | 12 |  | mA |  |
|  | Transmit Mode |  | 110 |  | mA |  |
| Crystal oscillator frequency |  |  | 32 |  | MHz |  |
| Modulation |  | FSK/LoRa |  |  |  |  |
| Frequency Range |  | 868 |  |  | MHz |  |
| Spread Frequency |  | 6 |  | 12 |  |  |
| Bandwidth |  | 125 |  | 500 | KHz |  |
| Effective Bitrate |  | 0.24 |  | 37.5 | kbps |  |
| Est. sensitivity |  | -117 |  | -137 | dBm |  |
| RF output power |  | 10 |  | 19 | dBm |  |
| Packet engine |  | 256 bytes with CRC |  |  |  |  |
| Transceiver |  | SX1272 |  |  |  |  |
| MCU |  | STM32F412RGT6 |  |  |  |  |

2. Pin Diagram

|  | Connected pin No. |  |  |  |
| --- | --- | --- | --- | --- |
| RF Board | SX1272 | STM32F412RGT6 | Pin definition | Description |
|  | 6 | 37 | RESET_sx | Reset trigger input |
|  | 9 | 29 | RF_DIO0 | Digital I/O |
|  | 10 | 28 | RF_DIO1 | Digital I/O |
|  | 11 | 27 | RF_DIO2 | Digital I/O |
|  | 12 | 26 | RF_DIO3 | Digital I/O |
|  | 13 | 25 | RF_DIO4 | Digital I/O |
|  | 14 | 24 | RF_DIO5 | Digital I/O |
|  | 17 | 21 | RF_SCK | SPI Clock input |
|  | 18 | 22 | RF_MISO | SPI Data output |
|  | 19 | 23 | RF_MOSI | SPI Data input |
|  | 20 | 20 | RF_NSS | SPI Chip select input |
| 1 |  | 57 | PB5 | GPIOB_PB5 |
| 2 |  | 58 | PB6/I2C1_SCL | I2C1_SCL |
| 3 |  | 59 | PB7/I2C1_SDA | I2C1_SDA |
| 4 |  | 61 | PB8/I2C1_SCL | I2C1_SCL |
| 5 |  | 62 | PB9/I2C1_SDA | I2C1_SDA |
| 6 |  | 1 | VBAT | VBAT+ |
| 7 |  | 2 | PC13 | GPIOC_PC13 |
| 8 |  | 8 | PC0/ADC1_10 | PC0_ADC1 |
| 9 |  | 9 | PC1/ADC1_11 | PC1_ADC1 |
| 10 |  | 10 | PC2/ADC1_12 | PC2_ADC1 |
| 11 |  | 11 | PC3/ADC1_13 | PC3_ADC1 |
| 12 |  | 14 | PA0/ADC1_0 | PA0_ADC1 |
| 13 |  | 15 | PA1/ADC1_1 | PA1_ADC1 |
| 14 |  | 16 | PA2/ADC1_2 | PA2_ADC1 |
| 15 |  | 12 | GND | GND |
| 16 |  | 19 | VDD-3.3V | VDD-3.3V |
| 17 |  | 33 | PB12/SPI2_NSS | SPI2_NSS |
| 18 |  | 34 | PB13/SPI2_SCK | SPI2_SCK |
| 19 |  | 35 | PB14/SPI2_MISO | SPI2_MISO |
| 20 |  | 36 | PB15/SPI2_MOSI | SPI2_MOSI |
| 21 |  | NC | NC | NC |
| 22 |  | 38 | PC7 | GPIO |
| 23 |  | 39 | PC8 | GPIOC_PC8 |
| 24 |  | 40 | PC9 | GPIOC_PC9 |
| 25 |  | 41 | PA8 | GPIOA_PA8 |
| 26 |  | 42 | PA9/USART1_TX | USART1_TX |
| 27 |  | 43 | PA10/USART1-RX | USART1-RX |
| 28 |  | 47 | GND | GND |
| 29 |  | 44 | PA11/USART6_TX | USART6_TX |
| 30 |  | 45 | PA12/USART6_RX | USART6_RX |
| 31 |  | 51 | PC10/USART3_TX | USART3_TX |
| 32 |  | 52 | PC11/USART3_RX | USART3_RX |
| 33 |  | 54 | PD2 | GPIOD_PD2 |
| 34 |  | 53 | PC12 | GPIOC_PC12 |
| 35 |  | 46 | JTMS-SWDIO | STM32 Download and debug port |
| 36 |  | 49 | JTCK-SWCLK |  |
| 37 |  | 50 | JTDI |  |
| 38 |  | 55 | JTDO-SWO |  |
| 39 |  | 56 | JTRST |  |
| 40 |  | 7 | NRST |  |
| 41 |  | 63 | GND | GND |

3. Size Dimension

4. Product model description

72

R  M

02

868

'868' means the Frequency Range

'00' means no MCU, other number means MCU No(or Sensor No)

Transceiver No, such as '72' means SX1272

Product type：module and end nodes

General division

MCU No. /Sensor No.

Transceiver No.

M for module & N for end nodes products

R means radio frequency products

5. Sample test results table

| TX Test |  |  |  |  |  | RX Test |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | Setting Power | Transmit Power | Transmit Current | Sens. |  | SF | Transmit Current. |
| 868MHz | 20 dBm | 18 dBm | 37 mA | -135 dBm |  | SF=12 | 11mA |
| standby current |  |  | 2mA | Frequency error |  | +4KHz |  |