W/DP RD-Q4187-01-A1

Customer Code :

DAPU  P/N:     O23B-L446-100.00MHz-A

Customer  P/N:

| DAPU |  |  | Customer Approval |
| --- | --- | --- | --- |
| Drew | Audited | Approved | Stamp, please! Thanks! |
| Date：2015.12.15 |  |  |  |

### Guangdong Dapu Telecom Technology Co.,Ltd

Bldg 16,.N.Ind.Zone,SSL Industry Park, Dongguan City, Guangdong Province, China

TEL：0086-0769-88010888  FAX：0086-0769-81800098

DATASHEET

Table of amendment

| Version | Revision contents | Prepared by | Revised date |
| --- | --- | --- | --- |
| 1.0 | The first issued | Amway | 2015.12.15 |

- Electrical Parameters

| MODEL：O23B-L446-100.00MHz-A |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Description |  | Parameters |  |  | Unit |  | Test Condition |  |  |
|  |  |  | Min. | Typ. | Max. |  |  |  |  |  |
| Output | Frequency |  | 100.00 |  |  | MHz |  |  |  |  |
|  | Output Waveform |  | Sine wave |  |  |  |  |  |  |  |
|  | Level |  | 6 |  |  | dBm |  |  |  |  |
|  | Load |  | 50 |  |  | Ω |  |  |  |  |
|  | Harmonics Suppression |  |  |  | -30 | dBc |  |  |  |  |
|  | Spurious Suppression |  |  |  | -60 | dBc |  |  |  |  |
| Frequency Stabilities | Frequency Tolerance vs. Operating Temperature Range |  | -5 |  | +5 | ×10-9 |  | TA varied from-20℃ to 70℃, measurement referenced to frequency observed with fref=(fmax+fmin )/2, Vcc=12.0V, Vc=2.5V, Oload=50Ω, temperature variable speed less than 2℃per minute. |  |  |
|  | Initial Frequency Tolerance |  | -0.3 |  | +0.3 | ×10-6 |  | Measurement referenced to frequency observed with TA=25℃, Vcc=12.0V, Vc=2.5V and after 15 minutes of operation, within 30 days after ex-works. |  |  |
|  | Frequency Tolerance vs. supply voltage |  | -5 |  | +5 | ×10-9 |  | measurement referenced to frequency observed TA=25℃, Vcc varied from 11.4V to 12.6V, Vc=2.5V, Oload=50Ω. |  |  |
|  | Frequency Tolerance vs. Load |  | -5 |  | +5 | ×10-9 |  | 5% Load Change Measurement referenced to frequency observed with TA=25℃, Vcc=12.0V, Vc=2.5V, Oload=50Ω. |  |  |
|  | Short Term Stability |  |  |  | 0.01 | ×10-9 |  | Temperature stability, no EMI\EMC or other interference, test after power for 1hour ref. to 25℃; 1s, using PN9000 equipment. |  |  |
|  | Aging Tolerance per day |  | -1 |  | +1 | ×10-9 |  | Vcc,Vc, TA constant Measurement referenced to frequency observed with TA=25℃, Vcc=12.0V, Vc=2.5V, Oload=50Ω and after 30 days of operation. |  |  |
|  | Aging Tolerance 1Year |  | -0.1 |  | +0.1 | ×10-6 |  |  |  |  |
| Power Supply | Supply Voltage |  | 11.4 | 12.0 | 12.6 | V |  |  |  |  |
|  | Steady Consumption |  |  |  | 300 | mA |  | @25℃ |  |  |
|  | Warm up current |  |  |  | 600 | mA |  |  |  |  |
| Voltage Control Characteristics |  | Frequency Tuning Range |  |  | -0.5 |  | ×10-6 | Vc=0V. measurement referenced to Vc=2.5V. |  |  |
|  |  |  | -0.3 |  | +0.3 |  | ×10-6 | Vc=2.5V. measurement referenced to exactly 100.00MHz. |  |  |
|  |  |  | +0.5 |  |  |  | ×10-6 | Vc=5.0V. measurement referenced to Vc=2.5V. |  |  |
|  |  | Linearity |  |  | 10 |  | % |  |  |  |
|  |  | Slope | Positive |  |  |  |  |  |  |  |
|  |  | Input Impedance | 100 |  |  |  | KΩ |  |  |  |
| Phase Noise |  | Phase Noise @25℃ |  | -110 | -105 |  | dBc/Hz | 10Hz |  |  |
|  |  |  |  | -140 | -135 |  |  | 100Hz |  |  |
|  |  |  |  | -155 | -150 |  |  | 1KHz |  |  |
|  |  |  |  | -160 | -155 |  |  | 10KHz |  |  |
|  |  |  |  | -175 | -170 |  |  | 100KHz |  |  |
|  |  |  |  | -175 | -170 |  |  | 1MHz |  |  |
| Environmental Conditions |  | Operable Temperature | -40 |  | +85 |  | ℃ |  |  |  |
|  |  | Storage Temperature | -55 |  | +105 |  | ℃ |  |  |  |
|  |  | ESD Level | Human Body Model, class2: 2000V to 4000V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |  |  |  |
|  |  |  | Machine Model, class B: 200V to 400V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |  |  |  |
|  |  | Moisture Sensitivity Level | Not humidity sensitive. |  |  |  |  |  |  |  |
|  |  | Vibration | Test Condition: 0.75mm ;acceleration:10g;10Hz～500Hz, one cycle per 30 min, test 2 hour. (3 times for each 3 directions X ,Y , Z)，IEC 68-2-06 Test Fc. |  |  |  |  |  |  |  |
|  |  | Shock | 50g；11ms；half sine wave (3 times for each 3 directions X ,Y, Z ),IEC 68-2-27 Test Ea/Severity 50A. |  |  |  |  |  |  |  |
| Full Package Storage |  | Relative humidity (%) | 20%～70% |  |  |  |  |  |  |  |
|  |  | Temperature (℃) | -10~35℃ |  |  |  |  |  |  |  |

2． Mechanical Structure (mm)

Note1:   Tolerance ± 0.20mm without mark

Note2:   The first two xx representative: week

After two xx representative: year

Note3:   Referential Weight 20.7g

- Test Circuit

- Reflow Soldering Curve (RoHS)

- Package(mm)