W/DP RD-Q4187-01-A1

Customer Code :

DAPU  P/N:      O11E-K316-40.00MHz

Customer  P/N:

| DAPU |  |  | Customer Approval |
| --- | --- | --- | --- |
| Drew | Audited | Approved | Stamp, please! Thanks! |
| Date：2015.12.11 |  |  |  |

### Guangdong Dapu Telecom Technology Co.,Ltd

Bldg 16,.N.Ind.Zone,SSL Industry Park, Dongguan City, Guangdong Province, China

TEL：0086-0769-88010888  FAX：0086-0769-81800098

DATASHEET

The Parameter of Product

Table of amendment

| Version | Revision contents | Prepared by | Revised date |
| --- | --- | --- | --- |
| 1.0 | The first issued | Amway | 2015.12.11 |

- Electrical Parameters

| MODEL： O11E-K316-40.00MHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Description |  | Parameters |  |  |  | Unit |  | Test Condition |  |
|  |  |  | Min. | Typ. | Max. |  |  |  |  |  |
| Output | Frequency |  | 40.00 |  |  |  |  | MHz |  |  |
|  | Output Waveform |  | HCMOS |  |  |  |  |  |  |  |
|  | Output Low Voltage |  |  |  | 0.4 |  | V |  | Vcc=3.3V, Oload=15pF |  |
|  | Output High Voltage |  | 2.4 |  |  |  | V |  | Vcc=3.3V, Oload=15pF |  |
|  | Duty Cycle |  | 40 | 50 | 60 |  | % |  | @50% |  |
|  | Rise / Fall Time (10%~90%) |  |  |  | 10 |  | ns |  |  |  |
|  | Load |  | 15 |  |  |  | pF |  |  |  |
| Frequency Stabilities | Frequency Tolerance vs. Operating Temperature Range |  | -0.01 |  | +0.01 |  |  | ×10-6 | TA varied from-40℃ to 85℃, measurement referenced to frequency observed with fref=(fmax+fmin )/2, Vcc=3.3V, Vc=1.65V, Oload=15pF, temperature variable speed less than 2℃per minute. |  |
|  | Initial Frequency Tolerance |  | -0.1 |  | +0.1 |  |  | ×10-6 | Measurement referenced to frequency observed with TA=25℃, Vcc=3.3V, Vc=1.65V, and after 15 minutes of operation, within 30 days after ex-works. |  |
|  | Frequency Tolerance vs. Supply Voltage |  | -5 |  | +5 |  |  | ×10-9 | measurement referenced to frequency observed TA=25℃, Vcc varied from 3.13V to 3.47V, Vc =1.65V and OLoad=15pF. |  |
|  | Frequency Tolerance vs. Load |  | -5 |  | +5 |  |  | ×10-9 | 5% load change measurement referenced to frequency observed with TA=25℃, Vcc=3.3V, Vc=1.65V, and OLoad=15pF. |  |
|  | Short-Term Stability: Allan Variance |  |  |  | 0.01 |  |  | ×10-9 | Temperature stability, no EMI\EMC or other interference，test after power for 1hour ref. to 25℃; 1s, using PN9000 equipment. |  |
|  | Aging Tolerance Per Day |  | -1 |  | +1 |  |  | ×10-9 | Vcc, Vc, TA constant measurement referenced to frequency observed with TA=25℃, Vcc= 3.3V, Vc=1.65V and after 30 days of operation. |  |
|  | Aging Tolerance 1 Year |  | -0.1 |  | +0.1 |  |  | ×10-6 |  |  |
| Power Supply | Supply Voltage |  | 3.13 | 3.3 | 3.47 |  |  | V |  |  |
|  | Steady Consumption |  |  |  | 350 |  |  | mA | @25℃ |  |
|  | Warm up current |  |  |  | 750 |  |  | mA |  |  |
| Voltage Control Characteristics |  | Frequency Tuning Range | -1 |  | -0.5 | ×10-6 |  |  | Vc=0V. measurement referenced to Vc=1.65V |  |
|  |  |  | -0.1 |  | +0.1 | ×10-6 |  |  | Vc=1.65V. measurement referenced to Exactly 40.00MHz |  |
|  |  |  | +0.5 |  | +1 | ×10-6 |  |  | Vc=3.3V. measurement referenced to Vc=1.65V |  |
|  |  | Linearity |  |  | 10 | % |  |  |  |  |
|  |  | Slope | Positive |  |  |  |  |  |  |  |
|  |  | Input Impedance | 100 |  |  | KΩ |  |  |  |  |
| Phase Noise |  | Phase Noise @25℃ |  | -110 | -100 | dBc/Hz |  |  | 10Hz |  |
|  |  |  |  | -130 | -120 |  |  |  | 100Hz |  |
|  |  |  |  | -140 | -135 |  |  |  | 1KHz |  |
|  |  |  |  | -145 | -140 |  |  |  | 10KHz |  |
|  |  |  |  | -150 | -145 |  |  |  | 100KHz |  |
|  |  |  |  | -150 | -145 |  |  |  | 1MHz |  |
| Environmental Conditions |  | Operable Temperature | -40 |  | +85 | ℃ |  |  |  |  |
|  |  | Storage Temperature | -55 |  | +105 | ℃ |  |  |  |  |
|  |  | ESD Level | Human Body Model, class2: 2000V to 4000V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |  |  |  |
|  |  |  | Machine Model, class B: 200V to 400V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |  |  |  |
|  |  | Moisture Sensitivity Level | Level 2. |  |  |  |  |  |  |  |
|  |  | Vibration | Test Condition: 0.75mm ;acceleration:10g;10Hz～500Hz, one cycle per 30 min, test 2 hour. (3 times for each 3 directions X ,Y , Z), IEC 68-2-06 Test Fc. |  |  |  |  |  |  |  |
|  |  | Shock | 50g; 11ms; half sine wave (3 times for each 3 directions X, Y, Z), IEC 68-2-27 Test Ea/Severity 50A. |  |  |  |  |  |  |  |
| Full Package Storage |  | Relative humidity (%) | 20%～70% |  |  |  |  |  |  |  |
|  |  | Temperature (℃) | -10~35℃ |  |  |  |  |  |  |  |

2． Mechanical Structure (mm)

Note1:   Tolerance ± 0.20mm without mark

Note2:   The first two xx representative: week

After two xx representative: year

Note3:   Referential Weight 8.0g

Note4:   NC is not connect

3．Test Circuit

4.  Reflow Soldering Curve (RoHS)

5.  Package: Tape & Reel (mm)