W/DP RD-Q4187-01-A1

Customer Code :

DAPU  P/N:     O22B-O428-100.00MHz-A

Customer  P/N:

| DAPU |  |  | Customer Approval |
| --- | --- | --- | --- |
| Drew | Audited | Approved | Stamp, please! Thanks! |
| Date：2015.12.10 |  |  |  |

### Guangdong Dapu Telecom Technology Co.,Ltd

Bldg 16,.N.Ind.Zone,SSL Industry Park, Dongguan City, Guangdong Province, China

TEL：0086-0769-88010888  FAX：0086-0769-81800098

DATASHEET

Table of amendment

| Version | Revision contents | Prepared by | Revised date |
| --- | --- | --- | --- |
| 1.0 | The first issued | Amway | 2015.12.10 |

- Electrical Parameters

| MODEL： O22B-O428-100.00MHz-A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Description | Parameters |  |  | Unit | Test Condition |
|  |  | Min. | Typ. | Max. |  |  |
| Output | Frequency | 100.00 |  |  | MHz |  |
|  | Output Waveform | Sine wave |  |  |  |  |
|  | Level | 8 | 13 |  | dBm |  |
|  | Load | 50 |  |  | Ω |  |
|  | Harmonics Suppression |  |  | -50 | dBc |  |
|  | Spurious Suppression |  |  | -80 | dBc |  |
| Frequency Stabilities | Frequency Tolerance vs. Operating Temperature Range | -0.05 |  | +0.05 | ×10-6 | TA varied from-40℃ to 75℃, measurement referenced to frequency observed with fref=(fmax+fmin )/2, Vcc=5.0V, Vc=2.25V, Oload=50Ω, temperature variable speed less than 2℃per minute. |
|  | Initial Frequency Tolerance | -0.1 |  | +0.1 | ×10-6 | Measurement referenced to frequency observed with TA=25℃, Vcc=5.0V, Vc=2.25V and after 15 minutes of operation, within 30 days after ex-works. |
|  | Frequency Tolerance vs. supply voltage | -5 |  | +5 | ×10-9 | measurement referenced to frequency observed TA=25℃, Vcc varied from 4.75V to 5.25V, Vc=2.25V, Oload=50Ω. |
|  | Frequency Tolerance vs. Load | -5 |  | +5 | ×10-9 | 5% Load Change Measurement referenced to frequency observed with TA=25℃, Vcc=5.0V, Vc=2.25V, Oload=50Ω. |
|  | Short Term Stability |  |  | 0.01 | ×10-9 | Temperature stability, no EMI\EMC or other interference, test after power for 1hour ref. to 25℃; 1s, using PN9000 equipment. |
|  | Aging Tolerance per day | -1 |  | +1 | ×10-9 | Vcc,Vc, TA constant Measurement referenced to frequency observed with TA=25℃,Vcc=5.0V, Vc=2.25V, Oload=50Ω and after 30 days of operation. |
|  | Aging Tolerance 1Year | -0.1 |  | +0.1 | ×10-6 |  |
| Power Supply | Supply Voltage | 4.75 | 5.0 | 5.25 | V |  |
|  | Steady Consumption |  |  | 400 | mA | @25℃ |
|  | Warm up current |  |  | 900 | mA |  |
|  | Warm-Up Time |  |  | 5 | min | @25℃within ±0.1×10-6 of final frequency with reference after 1 hour on. |
|  | Vref out |  | 4.5 |  | V |  |

| Voltage Control Characteristics | Frequency Tuning Range |  |  | -0.5 | ×10-6 | Vc=0V. measurement referenced to Vc=2.25V. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | -0.1 |  | +0.1 | ×10-6 | Vc=2.25V. measurement referenced to exactly 100.00MHz. |
|  |  | +0.5 |  |  | ×10-6 | Vc=4.5V. measurement referenced to Vc=2.25V. |
|  | Linearity |  |  | 10 | % |  |
|  | Slope | Positive |  |  |  |  |
|  | Input Impedance | 100 |  |  | KΩ |  |
| Phase Noise | Phase Noise @25℃ |  | -110 | -105 | dBc/Hz | 10Hz |
|  |  |  | -140 | -135 |  | 100Hz |
|  |  |  | -165 | -160 |  | 1KHz |
|  |  |  | -176 | -172 |  | 10KHz |
|  |  |  | -180 | -178 |  | 100KHz |
|  |  |  | -180 | -178 |  | 1MHz |
| Environmental Conditions | Operable Temperature | -40 |  | +85 | ℃ |  |
|  | Storage Temperature | -55 |  | +105 | ℃ |  |
|  | ESD Level | Human Body Model, class2: 2000V to 4000V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |
|  |  | Machine Model, class B: 200V to 400V; ANSI/ESDA/JEDEC JS-001-2010. |  |  |  |  |
|  | Moisture Sensitivity Level | Not humidity sensitive. |  |  |  |  |
|  | Vibration | Test Condition: 0.75mm ;acceleration:10g;10Hz～500Hz, one cycle per 30 min, test 2 hour. (3 times for each 3 directions X ,Y , Z)，IEC 68-2-06 Test Fc. |  |  |  |  |
|  | Shock | 50g；11ms；half sine wave (3 times for each 3 directions X ,Y, Z ),IEC 68-2-27 Test Ea/Severity 50A. |  |  |  |  |
| Full Package Storage | Relative humidity (%) | 20%～70% |  |  |  |  |
|  | Temperature (℃) | -10~35℃ |  |  |  |  |

2． Mechanical Structure (mm)

Note1:   Tolerance ± 0.20mm without mark

Note2:   The first two xx representative: week

After two xx representative: year

Note3:   Referential Weight 13.6g

- Test Circuit

- Reflow Soldering Curve (RoHS)

- Package(mm)