| # Non Conformance Report |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 26/03/2018 |  |  |  | NCR Number: |  | 14029 |  |
| Source: | Guangdong Dapu Telecom Technology Co. Ltd. |  |  |  | Part Number: |  | LFOCXO065337BULK |  |
| Description: | 32.7680MHz IQOV-70-1R ±10ppb -10 to 70C HCMOS |  |  |  | Sales Order Number: |  | 9046570 |  |
| Order Reference: | 9019468 |  |  |  | Warehouse: |  | LF |  |
| Batch Code: | 9048735 |  |  |  | Raised by: |  | Engineering |  |
| Inspection Cat: | 100 |  |  |  | Quantity Received: |  | 151 |  |
| Number Tested: | 50 |  |  |  | Number Rejected: |  | 1 |  |
| CPC Number: | 1 |  |  |  | Cust. Returns No: |  | n/a |  |
| Comment: |  |  |  |  |  |  |  |  |
| 50pcs tested 1 pcs has failed for having no output signal. Tests preformed under the following conditions: Temperature: 25.0°C ±2°C RH: 40 ~ 60 % These devices were tested as part of the standard Goods In control plan. 50pcs were tested using IQD’s Oscillator Bench test system. One device has failed; the device has failed for having no output signal. Further investigation of the fault on the device was carried out. The device was placed in the IQD Oscillator bench test system and left powered for fifteen minutes. After the warmup period the test sequence was run, and it is confirmed from the results that there is no output signal present. See attached results “9048735B” During the above tests the device was monitored using a Tektronix DPO oscilloscope TDS5054. The results of the output signal testing has been recorded using a Tektronix DPO oscilloscope TDS5054 and is attached to this report, see “NCR14029 LFOCXO065337 Output signal”. After completion of all of the testing it was found that the output of the device is indeed faulty. ENG 27/03/18 Please return to factory via Dapu FedEx account Y7444F to Guangdong Dapu Telecom Technology Co. Ltd. Building 16, Sci & Tech Industry Park, Northern Industrial Zone, SSL, Dong Guan, Guang Dong, P.R. China. Postcode: 523808 FAO: Cathy Guo Consequences: Consider and note the actual and potential If undetected at IQD, this 1pcs would have shipped to consequences of this failure the customer, therefore leading to a return for IQD, (examples – late and/or short delivery to the customer, potentially leading to a loss of reputation for IQD. rework costs, loss of margin, loss of reputation, etc) ------------------------------------------------------------------- |  |  |  |  |  |  |  |  |
| Corrective Action: |  |  |  |  |  |  |  |  |
| Department Actioned: |  |  |  | Guangdong Dapu Telecom Technology Co. Ltd. |  |  |  |  |
| Person Actioned: |  |  |  | Cathy Guo |  |  |  |  |
| Response Date Required: |  |  |  | ASAP |  |  |  |  |
| This part of the report is to be filled in and used as a Debit Note if the NCR is to be raised against the manufacturer. |  |  |  |  |  |  |  |  |
| Unit Cost: |  | $43.00 |  |  | Total Cost: | $43.00 |  |  |
| Additional Value: |  | - |  |  | Total Cost: | - |  |  |
| Qty to be Returned: |  | 1 |  |  | Priority: | 1 |  |  |
| Credit Only |  |  |  |  | Accept on Concession |  |  |  |
| Replace, Credit & Reinvoice |  | X |  |  | Other |  |  |  |
| Sub-Contractor Corrective Action: |  |  |  |  |  |  |  |  |
| IQD requests a full failure analysis detailing the issue with these parts, and a CAR explaining what measures will be put in place to prevent this failure from reoccurring. |  |  |  |  |  |  |  |  |